Top of Form

| U.S. Department of Transportation Federal Aviation Administration | Foreign Operations Specifications |  |
| --- | --- | --- |

C056. 	IFR Takeoff Minimums  - Airplanes					HQ Control:

HQ Revision:

Takeoff minimums are defined in 14 CFR Part 91.175(f) and hereinafter will be referred to as standard takeoff minimums.

a.  Takeoff minimums will be the highest of:

- published takeoff minimum in accordance with 14 CFR Part 97

- The following State of the Operator Authorized:

|  | Lowest RVR in feet (TDZ/MD/Rollout) | Airplane seating configuration or payload |
| --- | --- | --- |
| + |  |  |

b.  If the weather conditions at the airport of takeoff are below the foreign air carrier's landing minimums for that airport, the airplane may not depart from that airport unless an alternate airport for departure is designated and:

(1) the ceiling and visibility at the alternate airport at the time of departure, as well as the ETA at the alternate airport, is at or above the alternate minimums specified in OpSpec C055 of these operations specifications.

o  (2) For large airplanes, (seating configuration of more than 30 passengers or a maximum payload of more than 7,500 pounds), the alternate airport is located within one hour (1 hr) for two-engine aircraft and 2 hrs for aircraft with three or more engines from the airport of departure, at normal cruising speed in still air with one engine inoperative.

Note:  On some runways ceiling and visibility takeoff minimums are specified, (published in accordance with Part 97), and controlling.

o  (2) Large airplanes not authorized.

(3) For small airplanes, (seating configuration of 30 seats or less or a maximum payload capacity of 7,500 pounds or less), the alternate airport is located within one hour flying time of the departure airport at normal cruising speed in still air. The Touchdown Zone RVR report, if available, is controlling.

(4)  For small airplanes, when takeoff minimums are equal to or less than the applicable standard takeoff minimums, the foreign air carrier may use a takeoff minimum equal to the lowest authorized straight-in Category I landing minimum applicable to the foreign air carrier for that particular airport.

NOTE: Single-Engine IFR passenger-carrying operations are not authorized lower than standard takeoff minimums at any airport.  On some runways ceiling and visibility takeoff minimums are specified, (published in accordance with Part 97),  and controlling.

o  (2)  For large airplanes, (seating configuration of more than 30 passengers or a maximum payload of more than 7,500 pounds), the alternate airport is located within one hour (1 hr) for two-engine aircraft and 2 hrs for aircraft with three or more engines from the airport of departure, at normal cruising speed in still air with one engine inoperative.

(3)  For small airplanes, (seating configuration of 30 seats or less or a maximum payload capacity of 7,500 pounds or less), the alternate airport is located within one hour flying time of the departure airport at normal cruising speed in still air. The Touchdown Zone RVR report, if available, is controlling.

(4)  For small airplanes, when takeoff minimums are equal to or less than the applicable standard takeoff minimums, the foreign air carrier may use a takeoff minimum equal to the lowest authorized straight-in Category I landing minimum applicable to the foreign air carrier for that particular airport.

NOTE: Single-Engine IFR passenger-carrying operations are not authorized lower than standard takeoff minimums at any airport.  On some runways ceiling and visibility takeoff minimums are specified, (published in accordance with Part 97),  and controlling.

c.  The following limitations must be met:

(1) All takeoff operations based on RVR, must use RVR reports from the locations along the runway. For operations at or above RVR 1600 ft:

(a)  The touchdown zone (TDZ) RVR report, if available, is controlling.

- The mid RVR report may be substituted for an unavailable TDZ report.

NOTE: TDZ RVR used will be that at the beginning of the takeoff roll.  When a fourth Far End RVR value is reported it is advisory only.

o (2) Visibility or RVV 1/4 sm or TDZ RVR, 1600 ft, provided at least one of the following visual aids is available.

(a) Serviceable high intensity runway lights (HIRL);

(b) Serviceable runway centerline lights (CL);

(c) Visible runway centerline marking (RCLM); or

(d) In circumstances when none of the above visual aids are available, visibility or RVV ¼ statute mile may still be used, provided other runway markings or runway lighting provide pilots with adequate visual reference to continuously identify the takeoff surface and maintain directional control throughout the takeoff roll.

o (3)  For operations below RVR 1600 ft:

(a)  A minimum of two operative RVR reporting systems are required.

(b)  All available RVR reports are controlling.

o (4) RVR:  TDZ 1200 ft/mid, (if installed),1200 ft/ rollout 1000 ft, may be used provided RVR equipment and one of the following visual aids combinations are available: (a) Daylight Hours. Visible RCLM or serviceable: HIRL or CL lights. (b) Night Time Hours. Serviceable:  HIRL or runway CL lights.

o (5) RVR : TDZ 1000 ft/Mid, (if installed), 1000 ft/ rollout 1000 ft, may be used provided RVR equipment and one of the following visual aids combinations are available: (a) Serviceable CL lights, OR (b) serviceable HIRL and visible RCLM.

o (6) RVR: TDZ 600 ft /Mid, (if installed), 600 ft/Rollout 600 ft, may be used provided RVR equipment and ALL of the following visual aids are available: (a) serviceable HIRL. (b) Serviceable runway CL lights.

o (7) RVR:  TDZ 500 ft, mid, (if installed), 500 ft/ rollout 500 ft, may be used provided RVR equipment and ALL of the following visual aids are available. (a) Serviceable HIRL. (b) Serviceable runway CL lights.

⁯  d.

The foreign air carrier shall conduct all operations, in small airplanes, at or below 1600 RVR as described in subparagraph c above in compliance with the following limitations: (1) minimum flightcrew of  two pilots.  (2)Each pilot station must have operational equipment that displays a reliable indication of the following: (a) Aircraft pitch and bank, Aircraft heading, from a gyroscopic source, with each set of instruments driven by an independent power source; (c) Vertical speed; (d) Airspeed; and (e) Altitude. (3) For takeoffs when the RVR is less than Touchdown Zone RVR 1200 and Rollout RVR 1000, each airplane used must be operated at a takeoff weight which permits the airplane to achieve the performance equivalent to the takeoff performance specified for transport category aircraft.

1.	Issued by the Federal Aviation Administration.

2.	These Foreign Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:		Amendment Number:

4.	I hereby accept and receive the Foreign Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |