The Network Simulator NS-2

NIST add-on

Neighbor Discovery

July 26

# Table of Content

1	Table of Content	2

2	Overview	3

3	Design	3

3.1	Flow process	3

3.2	Support for different interfaces	4

3.3	Messages	4

3.4	Files location	4

4	TCL Configuration	4

4.1	Creation	4

4.2	Default values	4

4.3	Commands	5

# Overview

When moving from one network to another, the Neighbor Discovery module is used to create a new IP address in the new network. A new module has been created in NS-2 in order to provide the functionality of Neighbor Discovery. This new agent is implemented to support multiple types of interfaces (Ethernet, UMTS, 802.11..) by using broadcast or unicast messages depending on the technology.

Base Station

(AP+router)

Base Station

(AP+router)

RA

RA

RS

RS

Router

Router

Host

Host

Host

Host

Router

Router

RA

RA

RA

RA

Figure 1: Overview of ND in Ethernet and 802.11

Section 3 presents the design of the ND Agent implemented in NS2. Section 4 presents the TCL configuration.

# Design

## Flow process

The ND Agent implementation is based on the information provided by RFC 2461 Neighbor Discovery for IPv6, but only a few elements have been implemented.

If the node is a router, it will send unsolicited Router Advertisement (RA) messages periodically (following a law U[minRtrAdvInterval, maxRtrAdvInterval]).  If a router receives RA messages, it will silently discard them.  Upon reception of a Router Solicitation (RS), the router will decide if a reply can be sent or must be delayed depending on when the last RA was sent and when the next RA is scheduled.

A host can send RS messages to request the emission of an RA. The decision is made by an upper mobility management protocol.  Upon reception of an RA, the ND checks in its table if the prefix is already known. If it is not, then it creates an entry in the database and informs registered modules about the new prefix. Also using the information included in the RA message, a timer is used to determine when the prefix will expire if the node stops receiving the messages containing the same prefix. Every time a message arrives, it will update the prefix expiration time.

## Support for different interfaces

The ND Agent is located in all nodes but must be configured depending on the node’s type. In NS-2, some nodes (802.11) can generate broadcast packets while others do not have this capability (Ethernet, UMTS). To work around this problem, the ND Agent can be configured with a list of targets where it will send unicast messages.

## Messages

There are two types of messages implemented in the ND Agent: The Router Solicitation (RS) and the Router Advertisement (RA).

The RA contains 2 options:

- One prefix information

- One advertisement interval

## Files location

Coding and definitions of the Neighbor Discovery module are located in the following files:

- hsntg/nd.h: define the NDAgent

- hsntg/nd.cc: implementation of the NDAgent

- tcl/lib/ns-hsntg-nist.tcl: define default values for NDAgent

# TCL Configuration

## Creation

A utility method has been created to add the NDAgent into a node.

Use $node install-nd to add the ND module into a node (see tcl/lib/ns-hsntg-nist.tcl for implementation information).

## Default values

This section contains the attributes configured by TCL and their default values (defined in tcl/lib/ns-hsntg-nist.tcl)

| Name | Description | Default value |
| --- | --- | --- |
| router_ | Specify if the agent is located in the router or a host. | 0 |
| router_lifetime_ | The lifetime associated with the router (in seconds) | 5 |
| prefix_lifetime_ | The lifetime associated with the prefix (in seconds) | 5 |
| broadcast_ | Specify if the RA messages must be send in broadcast | 1 |
| minRtrAdvInterval_ | Minimum time between 2 RAs (in seconds) | 0.03 |
| maxRtrAdvInterval_ | Maximum time between 2 RAs (in seconds) | 0.07 |
| minDelayBetweenRA_ |  | 0.03 |
| maxRADelay_ | Maximum time for replying to an RS (in seconds) | 0.005 |
| default_port_ | The port number where the agent is attached in the node | 254 |

## Commands

The  following command are supported by the NDAgent

| Command | Arguments | Description |
| --- | --- | --- |
| start_ra | None | Start scheduling RA messages (for router only) |
| stop_ra | None | Stop scheduling RA messages (for router only) |
| send_rs | None | Send an RS (for host only) |
| dump-table | None | Dump the list of prefixes |
| set-minRtrAdv | Double | Set the minimum time between two RAs |
| set-maxRtrAdv | Double | Set the maximum time between two RAs |
| router-lifetime | Unsigned int | Set the router lifetime (in seconds) |
| prefix-lifetime | Unsigned int | Set the prefix lifetime (in seconds) |
| set-router | TRUE/FALSE | Specify if the agent must behave as a router or host |
| set-ifmanager | TCL reference | Set the interface manager that will receive notifications of new and expired prefixes |
| enable-broadcast | TRUE/FALSE | Set if the messages must be sent in broadcast or unicast |
| add-ra-target | NS Address | Add a target node for the RA messages |
| remove-ra-target | NS Address | Remove a target node for the RA messages |
| add-rs-target | NS Address | Add a target node for the RS messages |
| remove-rs-target | NS Address | Remove a target node for the RS messages |