SECTION 4.0

RWC BACM ECONOMIC FEASIBILITY METHODOLOGY

4.1	INTRODUCTION

This methodology is provided to enable serious PM-10 nonattainment areas to assess the economic feasibility of the measures described in sections 2.0 and 3.0 of this document.  Specifically, this section provides a methodology for estimating the cost and emission reduction effectiveness of each available integral and flexible measure.  The methodology does not provide specific data for agency and user costs associated with each measure since they are largely area-specific in nature and, hence, should be obtained locally.  For example, full-time employee (FTE) costs will vary from area-to-area, as will fuel costs and the purchase and installation costs for RWC devices.

4.2	INTEGRAL MEASURES

4.2.1  Public Awareness and Education

Emission Reduction Calculations:

As a general rule, no emissions reductions are generally associated with this measure, though a public awareness and education (PAE) program is vital to the overall success of any RWC control program.  It is typically very difficult to ascertain and quantify specific emission reductions that result from this program.  Therefore, no emission reduction methodology is given for this measure.

Cost Calculations:

The cost of implementing a BACM PAE program is more easily quantified and consists of the amount of time an FTE should spend to administer the program, plus any other costs such as radio, newspaper, and television advertising; establishment of an information hotline; education seminars; and the like.  The equations necessary to quantify these costs are given below.

A x B = C

where:

A  =	annual time for FTE to administer program, fraction

B  =	annual cost for an FTE, $

C  =	annual cost for FTE, $/year

The annual cost obtained above, C, is then added to the total cost of other program facets (e.g., advertising, hotlines, education seminars, etc.) to obtain the total annual cost.

C + D = E

where:

D  =	total annual cost of other program facets, $/year

E  =	total annual cost for PAE program, $/year

4.2.2  Mandatory Curtailment Program

Emission Reduction Calculations:

Unlike other RWC control measures in which PM-10 emission reductions are calculated on an annual basis, emission reductions from episodic curtailment are calculated on both an annual and worst case 24-hour basis.  This is because episodic curtailment is fundamentally a measure designed to reduce

24-hour emissions and, over the course of the heating season, implementation of this measure will also reduce annual emissions simply due to the cumulative effect of multiple

24-hour reductions.  Therefore, reductions on an annual and worst case 24-hour basis are calculated below.

Annual curtailment emissions are calculated based on the fraction of heating degree days (HDD) occurring during curtailment episodes divided by total HDD for the year multiplied by total yearly emissions.

The first step in this procedure is to calculate annual PM-10 emissions during the exceedance months using the following formula:

A x B/C = D

where:

A =	Annual baseline PM-10 RWC emissions, tons per year (tpy)

B =	Heating degree days during exceedance days

C =	Annual heating degree days

D =	Annual emissions of PM-10 during all exceedance days, tpy

The emission reductions are calculated as:

D x E = F

where:

E =	emission reduction credit level, fraction

F =	annual emissions reductions, tons PM-10 removed/year

Worst-case 24-hour emission reductions are calculated in a similar manner:

D24-hour x E = F24-hour

where:

D24-hour 	=	worst case 24-hour PM-10 emission, tons per day

F24-hour	=	24-hour emission reductions, tons, PM-10 removed/day

Cost Calculations:

The costs of implementing this program are calculated on both an annual and 24-hour basis.  These costs are divided into costs incurred by the implementing agency and the user.  The annual costs to the implementing agency are largely a function of the effort necessary to enforce and administer effectively a curtailment program.  The costs of a curtailment program to the wood stoves and fireplace user are the costs associated with heating through an alternative source.  These two cost elements (implementing agency and user) are determined separately as follows:

Implementing Agency Cost Calculations:  The annual cost of implementing a BACM program may be calculated by the formula below:

G x H = I

where:

G =	curtailment program cost, $/stove and fireplace●curtailment day

H =	number of curtailment days per year

I =	annual cost, $/year

The 24-hour implementing agency cost is obtained by simply dividing the annual cost, I, by the number of curtailment days per year, K.

I/K 	=	J24-hour

where:

J24-hour =	24-hour implementing agency incremental BACM cost, $/day

K	    =	number of curtailment days per year

User Cost Calculations:  The annual cost of the program for a stove user is based on an average cost incurred for each curtailment day.  This estimate accounts for the daily cost of an alternative heating system on curtailment days.  No costs to a fireplace user are estimated, since fireplaces are not intended to be used as a source of heat.

Lo =	M x K

where:

Lo =	average annual cost to each nonexempt stove user for the curtailment period,$/year

M  =	average cost to stove user per curtailment day,

$/day

The average cost to each stove user per curtailment day, M, may be difficult to precisely calculate.  A methodology for estimating this cost follows.

First, the cost of wood burning on curtailment days needs to be calculated.  This is estimated in a two-step process.  The first step yields the annual fuel cost to the wood stove owner as follows:

AC =	Cd x Ct

where:

AC  =	annual fuel cost to the wood stove owner, $/year

Cd  =	average number of cords burned in wood stoves, cords/year

Ct  =	average cost of a cord of wood, $/cord

This annual cost, AC, is then multiplied by the fraction of heating degree days during exceedance months divided by the annual heating degree days [(B/C), as calculated above], to obtain annual fuel costs during the curtailment period.  This product is then divided by the number of curtailment days per year to obtain the fuel costs per curtailment day:

FCD  =	[AC x (B/C)]/K

where:

FCD  =	fuel cost per curtailment day, $/day

The cost of alternative fuels needs to then be calculated.  This is based on the cost of obtaining an identical heat output (i.e., heat input x efficiency) from an alternative heating system.  Therefore, the heat output from a "typical" wood stove needs to be obtained.  A "typical" wood stove will vary from area-to-area, depending on the composition of the stove population.  Generally, surveys can be performed to estimate the typical heat output of wood stoves for a given area.  The energy input into a typical wood stove per curtailment day is based on the wood usage per day, which in turn, is obtained from the annual wood usage as follows:

DEI =	[Cd x EC x (B/C)]/K

where:

DEI =	daily energy input into typical wood stove, British thermal unit (Btu)/day

EC  =	energy content of a cord of wood, Btu/cord

This value, DEI, is then multiplied by the wood stove system efficiency to obtain the heat output on a daily basis.

DHO =	DEI x WSE

where:

DHO =	daily heat output of wood stove, Btu/day

WSE = 	typical wood stove efficiency, fraction

The daily heat output, DHO, can then be used to obtain the daily heat input requirements of an alternative system as follows.

DHR =	DHO/n

where:

DHR =	daily heat input requirement for alternative system, Btu/day

n   =	alternative system efficiency, fraction

The daily cost of this quantity of heat input is a function of the type of alternative system in the home (e.g., natural gas or propane heater, electric heat, etc.).  The cost of alternative heat on Btu basis [or kilowatt hour (kWh) basis for electricity] can be obtained from local gas or electric companies.  If electricity is used as the alternative heat, the energy units in Btu's must be converted to kWh as follows:

9.486 x 10-4 Btu = 2.778 x 10-7 kWh

As an example, the conversion of 100,000 Btu to kWh would be calculated as:

100,000 Btu x (2.778 x 10-7 kWh)/(9.486 x 10-4 Btu) =                                                    29.3 kWh

The fuel cost on a per day basis is then the difference between the alternative heating system fuel costs and the wood stove fuel heating costs as shown below:

M  =	(DHR x AEC) - FCD

where:

DHR =	daily heating requirement for alternative heat source, Btu/day

AEC =	alternative energy cost, $/Btu

The 24-hour cost to the stove user is simply M as defined above.

4.2.3  Measures to Improve Wood Burning Performance

4.2.3.1  Control of Wood Moisture Content.

Emission Reduction Calculations:

This measure reduces RWC emissions by promoting the burning of drier wood.  It is difficult to accurately quantify either emission reductions or costs from this measure due to factors that will necessarily vary from area to area.  However, the guidance document for residential wood combustion ("RWC Guidance Document") recommends that a maximum credit of 5 percent of baseline emissions be allowed for areas implementing this measure (Ref. 1).  For purposes of this document, a 5 percent credit is assumed. This credit can be calculated according to the formula below:

A x 0.05 = B

where:

A =	RWC annual baseline emissions, tons per year

B =	emissions reductions from control of wood moisture, tons per year

Cost Calculations:

In areas where this measure is not included in any

pre-serious area control strategy, the full cost of instituting this measure would be incurred. The cost to the implementing agency would include the purchase of wood moisture monitors and various administrative and educational charges.  The cost to the RWC owner would include the time and effort necessary to ensure the wood burned falls within an acceptable moisture content range.

4.2.3.2  Weatherization of Homes with Wood Stoves.

Emission Reduction Calculations:

The "RWC Guidance Document" suggests a credit for a weatherization program (Ref. 2).  The emission reductions from this measure are calculated using the formula below:

A x B = C

where:

A =	RWC annual baseline emissions, tons per year

B =	emission reduction credit

C =	emission reductions from weatherization, tons per year

Cost Calculations:

For this measure, the annual implementing agency costs are assumed to equal the product of the average cost of weatherizing a home multiplied by the number of homes weatherized per year.  There are minimal or no user costs assumed for this measure since it is intended to target low income households.

Implementing Agency Cost Calculations:  Total costs for this program are the product of the number of homes weatherized multiplied by the average cost to weatherize each home as shown below:

D x E = F

where:

D =	average cost to weatherize a home, $

E =	number of homes weatherized per year

F =	cost of weatherization program, $/year

A community may prefer to defer weatherization costs and implement the program over a longer time period as prescribed by EPA.  In that instance, the cost of the weatherization program should be amortized over that period.

Amortizing purchases is quite common and most people have experience with this through car and house purchases.  The technique described below is a amortizing method which yields equal payments incurred by the borrower over the life of the finance period.  The amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year time period (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor should be multiplied by the purchase cost to obtain the monthly payment over the length of the finance time:

E x CRF = G

where:

G =	implementing agency monthly payment for program, $

The monthly payment multiplied by 12 then yields the annual cost to the implementing agency:

F x 12 = Cm

where:

Cm =	annual cost to implementing agency, $/year

4.2.3.3  Educational Opacity Program.

Emission Reduction Calculations:

This measure reduces PM-10 emissions through an educational program identifying operation and maintenance habits contributing to visible emissions.  It is difficult to accurately quantify the emission reductions or costs from this measure due to lack of measurable parameters.  Therefore, a low credit (i.e., a maximum of 5 percent of baseline emissions) is recommended in the "RWC Guidance Document"

(Ref. 3).  For purposes of this document, the following formula can be used to estimate emission reductions from this measure:

A x 0.05 = B

where:

A =	RWC baseline emissions, tons per year

B =	emission reductions from educational opacity, tons per year

Cost Calculations:

In areas where this measure is not included in any

pre-serious area control strategies, the full cost of instituting this measure would be incurred, which would include opacity enforcement checks, public education forums and seminars, and related items.

4.2.4	All New Wood Stove Installations EPA-certified, Phase II Stoves or Equivalent

Emission Reduction Calculations:

Annual emission reductions are based upon the number of installations that occur in a given year of non-EPA-certified, Phase II stoves (or equivalent). The emission reductions are calculated based on the emission reduction credit associated with the conversion of a conventional to an EPA-certified, Phase II stove (or equivalent).  These reductions can be calculated using the formula below:

(A x B1 x B2 x C x D)/(454 x 2,000) = E

where:

A  	=	number of new installations of used, non-EPA-certified, Phase II stoves (or equivalent) per year

B1	=	emission reduction credit from conventional to Phase II or equivalent, decimal (Ref. 4)

B2	=	conventional wood stove emission factor, (Ref. 5) g/Kg

C  	=	average cords of wood burned in each stove per year, cords/year

D  	=	wood density, kg/cord

E	=	annual emissions reductions from this measure, tpy

454	=	conversion factor, grams to pounds, g/lb

2,000 =  conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Calculations:  To administer and enforce effectively this program, some cost will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce this program each year.  This cost can be estimated using the formula below:

F x G = H

where:

F =	time for an FTE to administer program, fraction

G =	annual cost for an FTE, $/year

H =	implementing Agency cost, $/year

User Cost Calculations:  To calculate the incremental cost to each stove user, the purchase and installation costs of an EPA-certified, Phase II stove (or equivalent) and of a conventional stove need to be obtained from local wood stove dealers.  The cost of the conventional stove is subtracted from the cost of an EPA-certified, Phase II stove (or equivalent).  This cost difference is assumed to be financed over a period of time by the stove buyer.  This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a amortizing method which yields equal payments incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

I x CRF = J

where:

I =	differential purchase and installation cost of stove, $

J =	monthly payment, $/month

The monthly payment multiplied by 12 then yields the annual cost:

J x 12 = Co

where:

Co =	annual cost to stove owner, $/year●stove

4.3	FLEXIBLE AVAILABLE MEASURES

4.3.1	Emission Reduction or Elimination - Existing Installations

4.3.1.1	Conversion of Existing Wood Burning Fireplaces to Gas Logs.

Emission Reduction Calculations:

Annual emission reductions are based on the number of wood-burning fireplaces converting to gas logs per year.  The first step is to obtain the number of fireplaces that will convert to gas logs under this measure for a given implementation period.  This implementation period is determined by the circumstances under which the nonattainment area is reclassified to serious.  This is done using the formula given below:

(A/X) = B

where:

A  =	number of existing fireplaces for baseline year

X  =	implementation period of available measure, years

B  =	annual number of fireplaces converting to gas logs

The emission reductions from this measure can then be calculated as follows:

(B x D x E x F)/(454 x 2,000) = G

where:

D	 =	fireplace emission factor, (Ref. 6) g/kg wood burned

E	 =	average wood usage in one fireplace, cords/year

F	 =	average wood density, kg/cord

G	 =	annual emission reductions from this measure, tpy

454	 =	conversion factor, grams to pounds, g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Effectiveness Calculations:  To administer and enforce effectively the program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce the program each year.  This cost can be estimated using the following formula:

H x I = J

where:

H =	time for an FTE to administer program, decimal

I =	annual cost for an FTE, $/year

J =	annual implementing agency cost, $/year

User Cost Calculations :  The costs to the fireplace user are based upon two components.  The first is the conversion cost of a conventional fireplace to gas logs, and the second is the difference in annual energy costs between wood and natural gas.

The cost to convert a conventional fireplace to a gas log fireplace will vary from community to community, but is not typically a big expense.  The local gas company and local wood stove and fireplace dealers should be contacted to obtain this cost.  Since this amount is not large, it is assumed the user will not finance this cost.

The energy cost difference between wood and natural gas is calculated assuming the fireplace usage (i.e., hours used per year) between a conventional and gas fireplace will not vary.  The fireplace usage can be calculated as shown below:

(E x F)/K = L

where:

K  =	fireplace burn rate, (Ref. 7) kg wood/hour

L  =	fireplace usage, hours/year

The amount of natural gas consumed during this time period is then calculated as follows:

L x M = N

where:

M  =	natural gas fireplace consumption rate, Btu/hour

N  =	energy consumption of natural gas fireplace, Btu/year

The annual cost difference between natural gas and wood is then calculated as shown below.  The cost of natural gas can be obtained from the local gas company, while the cost of wood can be obtained from wood stove dealers or local classified advertisements.

(E x O) - (N x P) = Q

where:

O  =	cost of a cord of wood, $/cord

P  =	cost of natural gas, $/Btu

Q  =	annual cost difference between natural gas and wood, $/year

The total annual cost of the user is then the difference of these two cost components:

R - Q = Co

where:

Co =	annual user cost $/year

R  =	cost to convert a conventional fireplace, $

4.3.1.2  Changeover to EPA-Certified, Phase II Stoves or

Equivalent.

Emission Reduction Calculations:

Annual emission reductions from this measure are based upon the number of conventional and non-EPA-certified stoves that convert annually to EPA-certified, Phase II stoves (or equivalent) minus the number of EPA-certified, Phase II stoves presently in the stove inventory.  The number of stoves annually converting to EPA-certified, Phase II stoves (or equivalent) is determined by the total number of stoves that will convert over the allowed implementation period.

To calculate the emission reductions that would occur through implementation of this measure, the formula below can be used:

[(A/X) x B1 x B2 x C x D]/(454 x 2000) = E

where:

A   	 =	number of affected stoves

B1	 =	emission reduction credit from conventional to Phase II or equivalent, decimal (Ref. 8)

B2	 =	conventional stove emission factor, (Ref. 9) g/Kg wood burned

C   	 =	average cords of wood burned in each wood stoves per year

D   	 =	wood density, kg/cord

E   	 =	emissions reductions from this measure, tpy

X   	 =	implementation period, years

454 	 =	conversion factor, grams to pounds, g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Calculations:  To effectively administer and enforce this program, some cost will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce this program each year.  This cost can be estimated using the formula below:

F x G = H

where:

F =	time for an FTE to administer program, fraction

G =	annual cost for an FTE, $/year

H =	implementing agency cost, $/year

User Cost Calculations:  To calculate the cost to each stove user, the purchase and installation cost of an EPA-certified, Phase II stove (or equivalent) needs to be obtained from local wood stove dealers.  For purposes of this analysis, it is assumed that the stove buyer will finance the purchase and installation cost over a period of time.  This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is an amortization method which yields equal payments incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

I x CRF = J

where:

I =	purchase and installation cost of stove, $

J =	monthly payment, $/month

The monthly payment multiplied by 12 then yields the annual cost:

J x 12 = Co

where:

Co =	annual cost to stove owner, $/year●stove

4.3.1.3	Changeover to Low-Emitting Devices.

Emission Reduction Calculations:

Annual emission reductions from this measure are based upon the number of conventional and non-EPA-certified stoves annually converting to low-emitting stoves minus the number of low-emitting stoves presently existing.  The number of stoves annually converting to low-emitting is determined by the total number of stoves that will convert over the allowed implementation period.

To calculate the emission reductions that would occur through implementation of this measure, the formula below can be used:

[(A/X) x B1 x B2 x C x D]/(454 x 2000) = E

where:

A   	 =	baseline number of conventional or Oregon 1986 certified stoves

B1	 =	emission reduction credit from conventional to low-emitting stove, decimal (Ref. 10)

B2	 =	conventional stove emission factor, (Ref. 11) g/Kg wood burned

C   	 =	average cords of wood burned in each wood stoves per year

D   	 =	wood density, kg/cord

E   	 =	emissions reductions from this measure, tpy

X   	 =	implementation period, years

454 	 =	conversion factor, grams to pounds, g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Calculations:  To administer and enforce effectively this program, some cost will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce this program each year.  This cost can be estimated using the formula below:

F x G = H

where:

F =	time for an FTE to administer program, fraction

G =	annual cost for an FTE, $/year

H =	Implementing Agency cost, $/year

User Cost Calculations:  To calculate the cost to each stove user, the purchase and installation cost of a low- emitting stove needs to be obtained from local wood stove dealers.  For purposes of this analysis, it is assumed that the stove buyer will finance the purchase and installation cost over a period of time.  This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is an amortization method which yields equal payments incurred by the borrower over the life of the finance period.   Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

I x CRF = J

where:

I =	purchase and installation cost of stove, $

J =	monthly payment, $/month

The monthly payment multiplied by 12 then yields the annual cost:

J x 12 = Co,s

where:

Co,s 	=	annual purchase and installation cost to stove owner, $/year●stove

To obtain the total costs to the user, the difference in fuel costs assuming an identical heat input between a conventional and low-emitting stove per year needs to be calculated (this example assumes a pellet stove for a low-emitting device).  The first step is to calculate the amount of heat input to a conventional stove per year.

C x K = L

where:

K 	=	heat content of wood, (Ref. 12) 106 Btu/cord

L	=	heat input to conventional stove, 106 Btu/year

The amount of pellet fuel this heat input represents is then calculated as shown below.

K/(0.008 x 2,000) = L

where:

0.008	=	heat content of pellet fuel, (Ref. 13) 106 Btu/lb

2,000	=	conversion factor, pounds to tons, lb/ton

L	   =	tons pellet fuel consumed per year, ton/year

The cost difference between pellet fuel and wood is then calculated.  The cost of pellet fuel and wood can be obtained through local wood stove dealers or through classified advertisements.

(L x M) - (B x N) = O

where:

M	=	cost of pellet fuel, $/ton

N	=	cost of wood, $/cord

O	=	annual cost difference between pellet fuel and wood, $/year

This cost is then added to Co,s to obtain the total annual user cost, Co.

Co,s + 0 = Co

where:

Co  =	total annual user cost, $/year

4.3.2  Emission Reduction or Emission Increase Prevention -

New Installations

4.3.2.1	Gas Fireplaces or Gas Logs in New Fireplace Installations.

Emission Reduction Calculations:

Annual emission reductions are based on the growth rate of fireplaces in the absence of regulations, assuming all new fireplaces installed are fueled by wood.  The calculations to obtain the emission reductions are given below.  First, the number of new gas logs and fireplaces installed per year needs to be calculated as shown below.

A x B = C

where:

A  =	annual wood-burning fireplace growth rate in absence of regulation

B  =	current wood-burning fireplace population

C  =	number of new wood-burning fireplaces installed per year in absence of regulation

The number of wood-burning fireplaces not installed annually due to this regulation is then assumed to equal C, the number of wood-burning fireplaces installed per year in the absence of regulation.  The emission reductions obtained from this measure are then calculated as follows:

(C x D x E x F)/(454 x 2,000) = G

where:

D	 =	fireplace emission factor, (Ref. 14) g/kg wood burned

E	 =	average wood usage in one fireplace, cords/year

F	 =	average cord density, kg/cord

G	 =	annual emission reductions, tons/year

454     =	conversion factor, grams to pounds, g/lb

2,000   =	conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Calculations:  To administer and enforce effectively the program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce the program each year.  This cost can be estimated using the following formula:

H x I = J

where:

H  =	time for an FTE to administer program, decimal

I  =	annual cost for an FTE, $/year

J  =	annual implementing agency cost, $/year

User Cost Calculations:  The costs to the user are based upon two components.  The first is the difference in the purchase and installation cost of gas logs or gas fireplaces and the purchase and installation cost of a conventional, wood-burning fireplace which could not be installed under this regulation.  These cost items can be obtained through local wood stove dealers.  This difference is assumed to be financed over the length of the home mortgage, since this cost is usually included with the rest of home purchase cost.

The second cost to the user is the difference in energy costs between gas and wood.  This cost difference is based on the amount of time a conventional fireplace would be used over the course of the year.  The calculations to obtain both of these costs are given below.

First, the cost difference between a gas fireplace and wood fireplace is calculated.

K - L = M

where:

K  =	cost of natural gas fireplace, $

L  =	cost of wood fireplace $

M  =	differential cost between gas and wood fireplace, $

The differential cost is the amortized over the length of the mortgage (which is typically 15 to 30 years) to obtain the annual differential cost, L.  This is calculated using an amortization factor called the capital recovery factor.  This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a amortization method which yields equal payments incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiplies the differential purchase cost, L, to obtain the monthly payment over the length of the finance time:

M x CRF = N

where:

N  =	monthly payment, $/month

The monthly payment multiplied by 12 the yields the annual cost for this cost component.

N x 12 = O

where:

O  =	annual cost differential between gas and wood and wood fireplace, $/year

The next step is to calculate the energy cost difference on an annual basis.  This is obtained based on the amount of time a fireplace is used and the corresponding natural gas and wood usage during this time.  First, the annual hours a conventional fireplace is used is calculated.

(E x F)/P = Q

where:

P  =	fireplace burn rate, (Ref. 15) kg wood/hour

Q  =	fireplace usage, hours/year

The amount of natural gas consumed during this time period is then calculated as follows:

Q x R = S

where:

R  =	natural gas fireplace consumption rate, Btu/hour

S  =	energy consumption of natural gas fireplace, Btu/year

The annual cost difference between natural gas and wood is then calculated as shown below.  The cost of natural gas can be obtained through the local gas company, while the cost of wood can be found in classified advertisements, or through wood stove dealers.

(E x T) - (S x U) = V

where:

T  =	cost of a cord of wood, $/cord

U  =	cost of natural gas, $/Btu

V  =	annual cost difference between natural gas and wood, $/year

The total annual cost to the user is then the sum of these two cost components:

O + V = Co

where:

Co  =	annual user cost, $/year

4.3.2.2  Upgrade Offset.

Emission Reduction Calculations:

The purpose of this measure is to prevent emission increases due to an increase in wood stove population.  As such, this measure does not reduce wood stove emissions and, therefore, emission reductions are considered negligible.  An emission reduction calculation for this measure is therefore not performed.  However, the cost incurred by the implementing agency and by the user can be estimated using the steps described below.

Cost Calculations:

Implementing Agency Cost Calculations:  To administer effectively this program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce this program each year.  This cost can be estimated using the formula below:

E x F = G

where:

E =	time for an FTE to administer and enforce this program, fraction

F =	annual cost for an FTE, $/year

G =	program cost, $/year

No implementing agency cost effectiveness for this measure is calculated.

User Cost Calculations:  To calculate the cost to each stove user, the purchase and installation cost of an EPA-certified, Phase II stove needs to be obtained from local wood stove dealers.  Note that for this measure, each new stove buyer may need to purchase an EPA-certified, Phase II stove.  Depending upon individual agreements between the new stove purchaser and the conventional stove owner (whose stove is to be upgraded), the cost to the new stove purchaser will vary, particularly if the conventional stove owner does not demand full payment for a newly installed EPA-certified, Phase II stove to decide to upgrade his or her stove.  It is therefore not possible to accurately quantify this cost, unless specific information between new stove purchasers and conventional stove owners is known.

For purposes of this analysis, it is assumed that the stove buyer will finance the purchase and installation cost over a period of time.  This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a amortization method which yields equal payments incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

H x CRF = I

where:

H =	cost to new stove purchaser,

I =	monthly payment, $/month

The monthly payment multiplied by 12 then yields the annual cost:

I x 12 = Co

where:

Co =	annual cost to stove owner, $/year●stove

4.3.2.3  Restriction in Number and Density of New Wood

Burning Stove and/or Fireplace Installations.

Emission Reduction Calculations:

Emission reductions from this measure should be based on an assumed growth rate of wood stoves and fireplaces minus stoves or fireplaces not installed due to the restriction.  This is done in two parts:  calculating wood stove emission reductions and calculating fireplace emission reductions.

Stove Emission Reduction:  The calculations to obtain the emission reductions from EPA-certified, Phase II stoves are given below:

A x B = C

where:

A =	annual EPA-certified, Phase II stove growth rate in absence of regulation, decimal

B =	current EPA-certified, Phase II stove population

C =	number of EPA-certified, Phase II stoves installed in absence of regulation

The number of stoves not installed due to this measure is dependent on the type of density restriction.  This may range from full restrictions on stoves (i.e., a ban) to partial restrictions.  If a full restriction program is instituted, the annual number of stoves not installed is simply the annual stove growth rate multiplied by the stove population.

For full restriction, use the following formula:

E = C

where:

E =	number of stoves not installed due to this measure

If a partial restriction on stoves in enacted, the number of stoves not installed will depend on the density allowance (e.g., number of stoves allowed per acre).  Historical data may be used to determine the average number of stoves per acre in the area, and the difference between this and the density restriction multiplied by the number of acres developed per year equals the number of stoves not installed.  The calculation to determine this value, E, is:

E = (D - RS) x AD

where:

E   =	annual number of stoves not installed due to this measure

D   =	average number of stoves per acre

RS  =	density restriction, number of stoves allowed per acre

AD  =	acres developed annually

The emission reductions obtained from not installing these stoves are calculated as shown below:

(E x F x G x H)/(454 x 2,000) = I

where:

F	 =	emission factor for EPA-certified, Phase II stoves, (Ref. 16) g/kg wood burned

G	 =	average wood burned in one stove per year, cords/year

H 	 =	average cord density, kg/cord

I 	 =	annual emission reductions from stoves, tons/year

454	 =	conversion factor, grams to pounds, g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

An identical procedure needs to be performed for fireplaces.

Fireplace Emission Reduction:  The calculations to obtain the emission reductions from fireplaces are given below:

AF x BF = CF

where:

AF =	annual fireplace growth rate in absence of regulation, decimal

BF =	current fireplace population

CF =	number of fireplaces installed in absence of regulation

The number of fireplaces not installed due to this measure is dependent on the type of density restriction.  This may range from full restrictions on fireplaces (i.e., a ban) to partial restrictions.  If a full restriction program is instituted, the annual number of fireplaces not installed is simply the annual fireplace growth rate multiplied by the fireplace population.

For full restriction, use the following formula:

EF = CF

where:

EF =	number of fireplaces not installed due to this measure

If a partial restriction on fireplaces in enacted, the number of fireplaces not installed will depend on the density allowance (e.g., number of fireplaces allowed per acre).  Historical data may be used to determine the average number of fireplaces per acre in the area, and the difference between this and the density restriction multiplied by the number of acres developed per year equals the number of fireplaces not installed.  The calculation procedure to determine this value, Ef, is:

EF = (DF - RSF) x ADF

where:

EF   =	annual number of fireplaces not installed due to this measure

DF   =	average number of fireplaces per acre

RSF  =	density restriction, number of fireplaces allowed per acre

ADF  =	annual developed acres

The emission reductions obtained from not installing these fireplaces are calculated as:

(EF x FF x GF x HF)/(454 x 2,000) = IF

where:

FF	 =	emission factor for fireplaces, (Ref. 17) g/kg wood burned

GF 	 =	average cord usage for fireplaces, cords/year

HF 	 =	average cord density for fireplaces, kg/cord

IF 	 =	annual emission reductions from fireplaces, tons/year

454	 =	conversion factor, grams to pounds, g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

Total emission reductions are then calculated as the sum of reductions from stoves and fireplaces.

H + HF = HT

where:

HT =	total emission reductions from this measure, tons/year

Cost Calculations:

Implementing Agency Cost Calculations:  To administer effectively this program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer the program each year.  This cost can be estimated using the formula below:

I x J = K

where:

I =	time for an FTE to administer program, fraction

J =	annual cost for an FTE, $/year

K =	annual implementing agency cost, $/year

User Cost Calculations:  There are two different procedures for calculating the cost to an RWC user, depending upon whether the RWC is a wood stove or fireplace.

The cost to each wood stove user is based on two items:  (1) the differential purchase and installation costs between a gas furnace or other form of heat and an EPA-certified, Phase II wood stove; and (2) the differences in fuel costs based on an identical heat input to a gas furnace and a wood stove.  Differential costs for fireplaces are estimated based on the cost of a wood-burning fireplace and a gas fireplace or gas log fireplace (both of which can be obtained from local wood stove and fireplace dealers) and the amount of time a fireplace is used annually.

Stoves:  The purchase and installation cost differential for stoves is calculated as:

L - M = N

where:

L =	purchase and installation cost for gas furnace, $

M =	purchase and installation cost for wood stove, $

N =	cost differential, $

For purposes of this analysis, it is assumed that this cost differential, N, is financed over the life of the stove which is typically 10 years.

This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  The technique described below is a method which yields equal payments incurred by the borrower over the life of the loan.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a ten year length of time:

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time (i.e., 120 months):

N x CRF = O

where:

N =	purchase and installation cost of stove, $

O =	monthly payment, $/month

To obtain the annual cost to the user, multiply this value, O, by 12 as shown below:

O x 12 = P

where:

P =	annual purchase installation cost to user, $/year

The difference in energy costs per year can be estimated once the costs for natural gas and wood are obtained on a $/106 Btu basis.  The cost of natural gas on a $/106 Btu basis can be obtained from the local gas company.  The energy cost of wood on a $/106 Btu basis can be calculated using the formula below.  The cost of wood ($/cord) can be obtained from local wood stove dealers or the classified advertisements.

Q/R =	S

where:

Q =	cost of wood, $/cord

R =	energy content of wood, 106 Btu/cord

S =	energy cost of wood, $/106 Btu

The differential cost is then obtained as:

(NGC - S) = T

where:

NGC =	natural gas cost, $/106 Btu

T =	cost differential, $/106 Btu

The annual cost difference is then calculated using the formula below:

GF x R x T = U

where:

U =	annual cost difference between natural gas and wood, $/year

The total annual cost to the wood stove user is the sum of these two components as shown below:

Cow = P + U

where:

Cow =	annual cost to wood stove user, $/year

Fireplaces:  The purchase and installation cost differential for fireplaces is calculated as shown below.  The cost for both types of fireplaces can be obtained from local wood stove dealers.  The cost of a gas line can be obtained from the local gas company.

LF - MF = NF

where:

LF =	purchase and installation cost for gas fireplace or gas log fireplace, $

MF =	purchase and installation cost for wood burning fireplace, $

NF =	cost differential, $

For purposes of this analysis, it is assumed that this cost differential, NF, will be financed over the length of the home mortgage.

This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a method which yields equal payments incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a ten year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

OF x CRF = PF

where:

OF =	purchase and installation cost of fireplace, $

PF =	monthly payment, $/month

To obtain the annual cost to the fireplace user, multiply this value, PF, by 12 as shown below:

PF x 12 = QF

where:

QF =	annual purchase and installation cost to fireplace user, $/year

The first step to obtain the differential fuel cost between a conventional fireplace and gas logs, is to calculate the annual hours a fireplace is used.  This is done according to the formula below.

(GF x HF)/RF = SF

where:

RF  =	fireplace burn rate, kg wood/hour (Reference 2)

SF  =	fireplace usage, hours/year

The amount of natural gas consumed during this time period is then calculated as follows:

SF  x TF  = UF

where:

TF  =	natural or propane gas fireplace consumption rate, Btu/hour

UF  =	energy consumption of natural or propane gas fireplace, Btu/year

The annual cost difference between natural gas and wood is then calculated as shown below.

(GF x VF ) - (UF x WF ) = XF

where:

VF  =	cost of a cord of wood, $/cord

WF  =	cost of natural gas $/Btu

XF  =	annual cost difference between natural gas and wood, $/year

The total annual cost to the fireplace user is the sum of XF and QF:

XF + QF = YF

where:

YF =	total annual cost for fireplace, $/year

4.3.2.4  Require New Wood Stove Installations be Low

Emitting.

Emission Reduction Calculations:

Emission reductions will be calculated based on reductions from EPA-certified, Phase II stoves to low emitting.

The emission reductions will be calculated assuming an annual stove growth rate in the absence of regulation, and the emission reductions achieved by requiring these new stoves to be low emitting.  This is done in a two step process.  First, the number of stoves that will be required to be low emitting is calculated.  Second, the emission reductions for these low- emitting stoves relative to EPA-certified, Phase II stoves are calculated as shown below:

A x B = C

where:

A =	stove growth rate in absence of regulation, decimal

B =	current stove population

C =	number of stoves restricted to low emitting

The number of stoves restricted to low emitting, C, is then used to calculate the emission reductions as follows:

(C x D1 x D2 x E x F)/(454 x 2,000) = G

where:

D1	 =	emission reduction credit for conversion from Phase II stove to low emitting, decimal (Ref. 18)

D2	 =	emission factor of EPA-certified, Phase II stove, (Ref. 19) g/Kg wood burned

E   	 =	average number of cords per year burned in one wood stove, cords/year

F   	 =	average cord density, kg/cord

G   	 =	emission reductions from this measure, tons/year

454 	 =	conversion factor, grams to pounds, g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Calculations:  To administer and enforce effectively the program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer and enforce this program each year.  This cost can be estimated using the formula below:

H x I = J

where:

H =	time for an FTE to administer and enforce program, fraction

I =	annual cost for an FTE, $/year

J =	annual implementing agency cost, $/year

User Cost Calculations:  To calculate the cost to each stove user, the purchase and installation cost of a low- emitting stove needs to be obtained from local wood stove dealers.  For purposes of this analysis, it is assumed the stove buyer will finance (i.e., discount) this cost over a period of time.  The purchase and installation of a low- emitting stove may be obtained by calling local dealers.

Financing stove costs requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a method which yields equal payment incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time:

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time (i.e., 120 months):

K x CRF = L

where:

K =	purchase and installation cost of stove, $

L =	monthly payment, $/month

To obtain the annual cost to the user, multiply this value, K, by 12 as shown below:

L x 12 = M

where:

M =	annual cost of purchase and installation of a low- emitting stove, $/year

In addition to the purchase and installation cost, the annual fuel cost must be calculated.  This cost is based on the difference between the cost of pellet fuel and wood, assuming the same heat input to a pellet stove and an EPA-certified, Phase II stove (this example assumes a pellet stove for a low-emitting device).  The calculation procedure is given below.  First, calculate the annual heat input to an EPA-certified, Phase II stove as shown:

E x N = O

where:

N =	average cord energy content, Btu/cord

O =	total annual heat input to stove, Btu/year

Once this value is calculated, the annual cost of pellet fuel can be obtained using the following formula.  The cost of pellet fuel ($/ton) can be obtained through local wood stove dealers.

(N x P x Q)/(8,000 x 2,000) = R

where:

P   	=		cost of pellet fuel, $/ton pellets

Q	=		relative efficiency ratio of EPA-certified, Phase II wood stoves to low-emitting stoves, decimal (Ref. 20) (e.g., pellet stoves in this example)

R	=		annual fuel cost of pellets, $/year

2,000  =	conversion factor, pounds to tons, lb/ton

8,000  =     heat content of pellet fuel, Btu/lb

The annual cost of wood is calculated according to the formula below.  The cost of wood ($/cord) can be obtained through wood stove dealers or classified advertisements.

E x S = T

where:

S =	cost of wood, $/cord

T =	annual wood cost, $/year

The annual wood cost, T, is then subtracted from the annual pellet cost, P, to obtain the incremental fuel costs of switching to pellet fuel from cord wood as shown below:

Q - T = U

where:

U =	incremental fuel cost, $/year

The total annual costs to the stove purchaser are then calculated as:

M + U = Co

where:

Co =	total annual cost to stove purchaser, $/year●stove

4.3.3	Emission Reduction - New and Existing Installations

4.3.3.1  Device Offset.

Emission Reduction Calculations:

Emission reductions will be based on a decrease in emissions from removal of an appropriate number of conventional stoves for every stove installed.

The calculations to obtain the emission reductions are given below.  First, the number of new stoves installed per year needs to be calculated as shown below.

A x B = C

where:

A =	annual stove growth rate in absence of regulation

B =	current stove population

C =	number of new stoves installed per year

The emission reductions can then be calculated as:

(C x D1 x D2 x E x F)/(454 x 2,000) = G

where:

D1	 =	emission reduction credit from conventional to Phase II stove or equivalent, decimal (Ref. 21)

D2	 =	emission factor for conventional stove, (Ref. 22) g/Kg wood burned

E 	 =	average wood burned in one stove per year, cords/year

F 	 =	average wood density, kg/cord

G 	 =	annual emission reductions, tons/year

454	 =	conversion factor, grams to pounds,g/lb

2,000 =	conversion factor, pounds to tons, lb/ton

Cost Calculations:

Implementing Agency Cost Calculations:  To effectively administer the program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer the program each year.  This cost can be estimated using the following formula:

H x I = J

where:

H =	time for an FTE to administer program, decimal

I =	annual cost for an FTE, $/year

J =	annual implementing agency cost, $/year

The implementing agency cost effectiveness is calculated as the ratio of program costs divided by the total reductions obtained.

CEia =	J/G

where:

CEia =	implementing agency cost effectiveness, $/ton

PM-10 removed

User Cost Calculations:  The costs to stove purchasers proposing to install a new EPA-certified, Phase II stove in the serious nonattainment area are assumed to equal the installation cost of a new EPA-certified, Phase II stove, the cost to remove a conventional stove, and the cost of an alternative heating system to replace the conventional stove.  These costs can be obtained from local wood stove dealers, the local gas company, and the local electric company.  For purposes of this analysis, it is assumed the purchaser will finance these costs over a period of time.

This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a method which yields equal payments incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

K x CRF = L

where:

K =	total cost to remove conventional stove, plus cost of alternative heating system, plus cost of EPA-certified, Phase II stove, $

L =	monthly payment, $

The monthly payment multiplied by 12 then yields the annual cost:

L x 12 = Co

where:

Co =	annual cost to stove owner, $/year●stove

4.3.3.2  Upgrade Offset.

Emission Reduction Calculations:

Emission reductions are calculated assuming a stove growth rate in the absence of regulation with an appropriate number of conventional stoves upgraded for every new EPA-certified, Phase II stove installed.  The first step in calculating the emission reductions is to estimate the number of stoves that will be upgraded using the two formulas below:

A x B = C

where:

A =	stove growth rate in absence of regulation, decimal

B =	current stove population

C =	number of stoves installed

The number of upgraded stoves is calculated as:

D =	E x C

where:

D =	number of conventional stoves upgraded

E =   appropriate number of stoves to achieve net

emission reduction

The emission reductions are calculated in a two-step process.  First, the reductions from the upgraded stoves relative to conventional stoves are estimated.  Second, the emissions from the new stoves installed are calculated and subtracted from the reductions obtained in the first step.

(D x E1 x E2 x F x G)/(454 x 2000) = H

where:

E1 	 =	emission reduction credit from conventional to Phase II, decimal (Ref. 23)

E2	 =	emission factor for conventional stoves, (Ref. 24) g/Kg wood burned

F   	 =	average cords per year burned in one stove

G   	 =	average cord density, kg/cord

H   	 =	emission reductions from upgraded stoves, tons/year

454 	 =	conversion factor, grams to pounds, g/lb

2,000   =	conversion factor, pounds to tons, lb/ton

The emissions increase resulting from installation of that new stove are calculated as follows:

(C x I x F x G)/(454 x 2000) = J

where:

I   =	emission factor for EPA-certified, Phase II stoves, (Ref. 25) g/kg wood burned

J   =	emissions from new stove installations, tons/year

The emission reductions are then calculated as the difference between the reductions from the upgraded stoves minus the emissions occurring from the new stoves as shown below:

K = H-J

where:

K =	emission reductions from this measure, tons/year

Cost Calculations:

Implementing Agency Cost Calculations:  To effectively administer this program, some costs will be incurred.  These costs may be estimated based on the amount of time an FTE would spend to administer this program each year.  This cost can be estimated using the formula below:

E x F = G

where:

E =	time for an FTE to administer program, fraction

F =	annual cost for an FTE, $/year

G =	Implementing Agency cost, $/year

User Cost Calculations:  To calculate the cost to each stove user, the purchase and installation cost of an EPA-certified, Phase II stove needs to be obtained from local wood stove dealers.  Depending upon individual agreements between the new stove purchaser and the conventional stove owners (whose stoves are to be upgraded), the cost to the new stove purchaser will vary, particularly if the conventional stove owners do not demand full payment for a new installed EPA-certified, Phase II stove.  It is therefore not possible to accurately quantify this cost, unless specific information between new stove purchasers and conventional stove owners is known.

For purposes of this analysis, it is assumed that the stove buyer will finance whatever costs are incurred over a period of time.  This requires that these costs be "amortized" by a numerical factor based on the length of the finance period and the interest rate.  The technique described below is a method which yields equal payment incurred by the borrower over the life of the finance period.  Amortizing purchases is quite common and most people have experience with this through car and house purchases.  This amortization factor is called the capital recovery factor and is calculated as follows:

CRF =	[i/12(i + i/12)n]/[(1 + i/12)n-1]

where:

CRF =	capital recovery factor, decimal

i   =	annual real interest rate, decimal

n   =	length of finance time, months

As an example, the CRF can be calculated using a 10 percent real interest rate and a 10-year length of time (i.e., 120 months):

CRF =	[0.1/12 (1 + 0.1/12)120]/[(1 + 0.1/12)120-1] = 0.01322

This factor multiples the purchase cost to obtain the monthly payment over the length of the finance time:

H x CRF = I

where:

H  =	cost to new stove purchaser,

J  =	monthly payment, $

The monthly payment multiplied by 12 then yields the annual cost:

I x 12 = Co

where:

Co =	annual cost to stove owner, $/year●stove

References

1.	U.S. Environmental Protection Agency, Guideline Series.  "Guidance Document for Residential Wood Combustion Emission Control Measures."  EPA-450/2-89-015.  September 1989.  pp. F1 - F4.

2.	Reference 1, pp. F1 - F4.

3.	Reference 1, pp. F1 - F4.

4.	Reference 1, pp. F1 - F4.

5.	U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards, Compilation of Air Pollutant Emission Factors (AP-42), Research Triangle Park, North Carolina.

6.	Reference 5.

7.	Reference 5.

8.	Reference 1, pp. F1 - F4.

9.	Reference 5.

10.	Reference 1, pp. F1 - F4.

11.	Reference 5.

12.	Reference 1, p. A8.

13.	Hearth and Home.  August 1990.  p. 25.

14.	Reference 5.

15.	Reference 5.

16.	Reference 5.

17.	Reference 5.

18.	Reference 1, pp. F1 - F4.

19.	Reference 5.

20.	Reference 1, pp. F1 - F4.

21.	Reference 1, pp. F1 - F4.

21.	Reference 6.

22.	Reference 5.

23.	Reference 1, pp. F1 - F4.

24.	Reference 5.

25.	Reference 5.

The credit should reflect the types of stoves it is assumed would be installed in the absence of the restriction (e.g., conventional stoves and EPA-certified, Phase I stoves).