- CFLHD SURVEY CONTROL SHEET SPECIFICATIONS

I	Prepare Survey Control Sheet(s) to be included with the PS&E Package. The purpose of the Survey Control Sheet is to list the coordinates, pertinent metadata, and graphically show the location of all Primary (2000 & 3000 Series) and Supplemental (5000 Series) Control Monuments for a particular project. Survey Control Sheets include the following:

(a)	Use the standard FHWA border;

    - Place text in accordance with the CFLHD CADD Standards Manual, Chapter 5 – Annotation Standards and Specifications;

    - Place the following information in the upper left hand portion of the sheet:

      - The Project Designation and Project Name;

      - Date(s) of Field Work, by whom;

      - Date of Final Adjustment, by whom;

      - Project Units (Horizontal & Vertical);

      - Project Coordinate System:

        - State Zone;

        - Horizontal Datum (Epoch);

        - Geoid Model; and

        - Statement describing the horizontal control monument(s) and the datum used as a basis for the establishment of coordinate values;

      - Vertical Datum:

        - A statement describing the vertical control monument(s) and the datum used as a basis for the establishment of elevation values; and

      - The GPK file, Chain(s) used to define the Alignment,

    - Place the following information in the upper right hand portion of the sheet:

      - See FP-03, Sections 107.02 and Section 152 for Surveying and Staking Requirements. Set permanent monuments according to FP-03, Section 621.

    - A table listing:

      - Point Number;

      - Northing;

      - Easting;

      - Elevation;

      - Latitude;

      - Longitude;

      - Ellipsoid Height;

      - Mapping Angle;

      - Combined Factor;

      - Station and Offset from the final alignment and;

      - Description of Monument and stampings.

Note: Northing and Easting coordinate values will be State Plane Grid Coordinates.

    - Place the following information in the lower right hand portion of the sheet:

      - The creation date of the Control Data Sheet and the initials of the responsible individual(s) and;

      - Modification dates of the Control Data Sheet and the initials of the responsible individual(s).

    - Place the following statement in the lower left hand portion of the sheet;

      - Note: To precisely check distances between points as measured on the ground: inverse the State Plane Coordinates and divide the computed distance by the mean Combined Factor of the two points;

      - To compute Geodetic Azimuth, use the following formula: Geodetic Azimuth = Grid Azimuth + Convergence Angle.

    - Place an open triangle symbol (T-Point cell), centered on the stated coordinate values of the all Control Monuments, within the drawing. The open triangle symbol will have a printed height of 0.10”. Include the point number of the Control Monument in the drawing accordance with the CFLHD CADD Standards Manual, Chapter 5 – Annotation Standards and Specifications.