Use of these instructions with a residential data collection checklist assumes a general understanding of the provisions of the 2012 International Energy Conservation Code and key concepts and definitions applicable to those provisions. It may be necessary to consult the 2012 IECC and relevant support materials when in doubt about a particular item in the checklist. Each checklist item contains the corresponding residential provisions of the 2012 IECC code section(s) for quick reference. While most of the code provisions are included in the checklists, there are a few requirements that are deemed administrative and/or without significant impact, and these are not included. Some code requirements have exceptions and/or exemptions, many of which are not included in the checklist text but are described in these instructions.

The checklists were originally developed for use in addressing Recovery Act and State Energy Program requirements, both of which are focused on reducing energy consumption in buildings. However, they can be useful inspection tools for all code officials in jurisdictions that have adopted the 2012 IECC, noting that modifications may be necessary where adopting jurisdictions have amended the IECC.

The checklists are divided into stages corresponding to traditional building inspection stages. A building may require more than one field visit to gather compliance data during each stage of construction. Data from multiple buildings can be combined to derive a single building evaluation. This may occur where multiple buildings are being simultaneously constructed, with construction in varying stages occurring at the same time (e.g., a housing subdivision, condominium or apartment complex). In these cases, the same building should be used for at least one complete inspection stage (i.e., plan review, foundation, framing, insulation, or final inspection). Additionally, the buildings must be of the same building type (e.g. single family, modular, multifamily, townhouse, etc.).

Completing the General Information Section. All inputs at the top of the first page of the checklist should be completed. Some of these inputs are repeated at the start of each stage of construction. Where a single building is being evaluated for each stage of construction, the duplicate inputs can be ignored. Where different buildings are used for completing different stages of construction, the top portion of each checklist stage must be completed for each different building evaluated.

- Compliance Approach: Compliance with the energy code can be demonstrated through a prescriptive, trade-off, or performance approach. In evaluating building compliance, the prescriptive approach should be assumed unless documentation is obtained from the building department or responsible authority demonstrating compliance with either the trade-off or performance approach. The Prescriptive Code Value column on the checklist contains the prescriptive requirement which must be met under the prescriptive approach. If a trade-off or performance approach is used to demonstrate compliance, the buildings may not comply with one or more of these prescriptive values and yet may still be deemed to comply with the code, in which case the checklist requirement should be marked as compliant on the basis that some other aspect of the building exceeds the code. For example, assume a trade-off approach was used and a valid worksheet or software report was submitted showing a compliant building in Climate Zone 4 with R-5 continuous basement insulation. In Climate Zone 4, the code’s prescriptive insulation R-value requirement for a basement wall is listed as R-10. In this example, the basement insulation should be marked as compliant even though it does not meet the prescriptive requirement given on the checklist. If documentation is submitted that indicates compliance via a valid trade-off approach, there will be some other building component that exceeds code requirements and offsets the non-compliant basement wall. Values that differ from prescriptive but that are determined during plan review to comply based on an approved alternative compliance approach should be noted in the Plans Verified Value column of the checklist.

- Building Type: Select the type of residential building to which the checklist applies. For multifamily residential buildings, where multiple dwelling units are similar, compliance evaluation is not required for all code requirements in all dwelling units. Where units are alike, one or two units may be enough to determine compliance based on other units having similar construction. Where multiple floors are identical, many code requirements can be evaluated on only one floor of the building, and be considered representative of the others. However, the ground floor and top floor (roof) assemblies will also need to be evaluated.

- Foundation Type: Select the foundation type(s). Where multiple foundation types are present in the building, select all that apply.

- Project Type: Select the project type. Only one project type can be selected for a given checklist. If a building has both an addition and a renovation, complete one checklist for the addition and another for the renovation.

Complies Column. Each checklist item must be selected as compliant (Y), not compliant (N), not observable (N/O), or not applicable (N/A). N/O and N/A options are intended for very different circumstances. N/A is a valid option for any code requirement that does not apply to a given building (e.g. sunroom or swimming pool requirements in a residence that does not have these features). N/O is an option for cases where it is impossible to evaluate a code requirement on a given building because it is hidden or otherwise unavailable for observation at the time a compliance inspection is made. Evaluators should limit the use of the N/O option. The BECP evaluation methodology was designed as a random sampling of buildings, and if a large number of code requirements are marked N/O, the evaluation methodology no longer provides a valid observation of an entire building. However, it is acceptable to use multiple buildings of the same type but in different stages of construction to comprise a single building sample. When evaluating a renovation or addition, it is appropriate to select N/A for code provisions that do not apply to the specific renovation or addition. N/A should not be selected for cases where the code provision cannot be inspected because it has been covered or can’t be observed – instead select N/O. It should be noted that adopting jurisdictions (state or local government) may amend the IECC and/or enforcing authorities (code officials and inspectors) may have developed interpretations of the code that might result in minor modifications to code requirements. As an example, the requirement that a certificate identifying the energy-related features of the building be placed in the electrical box might be modified to allow its placement elsewhere in the building. In cases where these minor alterations are deemed by the evaluator to still meet the intent of the code, the checklist item can be marked as compliant with a corresponding comment from the evaluator.

There may be cases where the building, as constructed, does not match the originally submitted compliance documentation and building plans. The documentation may demonstrate compliance, but the as-built building does not comply. Alternatively, the documentation may show non-compliance, yet the building actually does comply. In such cases, the building should be evaluated according to how it was actually constructed. If a code requirement is met in the as-built building, that requirement should be marked as compliant, even if the documentation indicates it is not compliant. If the documentation indicates the building does comply, but an in-field inspection indicates it doesn’t, the applicable code requirement(s) should be marked non-compliant.

A similar question might arise for a code requirement that is found to be non-compliant by an evaluator, but the non-compliance was also observed by a code official who records it as part of the field inspection results and subsequently confirms that the non-compliance was addressed and the final construction does comply. In this situation, the code requirement should still be marked as non-compliant unless the evaluator visually inspects the item in the field to verify that it was fixed.

Plans Verified Value Column. Evaluators should review the submitted plans and specifications for the building prior to an onsite inspection. During this review, the evaluator can use the Plans Verified Value column to record values found on the plans and specifications. This column is critical if a building complies by a trade-off or performance approach, as opposed to a prescriptive approach. The checklists already provide the prescriptive requirements in the Prescriptive Code Value column. However, alternative compliance approaches may indicate values that differ from the code’s prescriptive values, but because of the use of a trade-off or performance approach, they are now the prescriptive values for that particular building. Recording those values in the Plans Verified Value column enables the evaluator to easily check against the correct values during field inspection of the building.

As an example, basement, crawl space wall, and slab-on-grade insulation is not required in some climate zones. The prescriptive R-value is listed as R-0 on the checklists for these locations. But a trade-off or performance approach may have indicated insulation would be installed on the foundation, in lieu of some other component being insulated to a lesser degree than required prescriptively by the code. In this case, the foundation insulation R-value and depth of insulation should be recorded in the Plans Verified Value column so that the inspector knows what is required for that particular building and can look for it during field inspection.

Field Verified Value Column. The checklists are used to collect relevant information about the building as well as to verify compliance. Provide the value (R-value, U-factor, depth of insulation, etc) observed in the actual building while in the field in the Field Verified Value column. In many cases, more than one value may be observed, in which case all values observed should be recorded. For example, windows in the building may have a different U-factor than sliding glass doors. Both should be recorded along with the building component to which they correspond. How compliance is determined when multiple values are found may vary depending on the compliance approach:

- Prescriptive Approach – Insulation R-values: All insulation R-values must be equal to or greater than the prescriptive code value. Enter all observed R-values into the Verified Value column, or the Comments/Assumptions column if more space is needed. If any are less than the prescriptive R-value, this checklist item is deemed to fail.

- Prescriptive Approach – Fenestration U-factors and SHGC: Enter all observed U-factors into the Field Verified Value column. If all values are less than or equal to the code value, the checklist item is deemed to pass. Where multiple U-factors are observed, and some are above and some below the code value, it may be necessary to check the area-weighted average, which will require fenestration areas. If the area-weighted average glazing U-factor is less than or equal to the prescriptive code value, then the checklist item is deemed to pass. The areas, U-factors, and calculations can be provided in the Comments/Assumptions area of the checklist or on a separate worksheet. A similar approach should be taken for fenestration SHGC. Note that up to 15 ft2 of fenestration can be exempted from the prescriptive U-factor and SHGC requirements, and one side-hinged door up to 24 ft2 can be exempted from the prescriptive door U-factor requirements.

- Trade-Off and Performance Approaches: Under alternative approaches, the values and areas to be verified are those on the compliance documentation. Where multiple values are observed, enter the observed R-values, U-factors, and their corresponding areas into the Field Verified Value column if space permits. Where space does not allow this, use the Comments/Assumptions area of the checklist or a separate worksheet.

Some code requirements can be subjective, and evaluating compliance against those requirements becomes a judgment call. The checklists contain a grading scale to help evaluators address subjective compliance determinations. For such requirements, evaluators may mark a code requirement as compliant, but then are further asked if the installation quality is rated Good, Fair, or Poor. This grading scale aligns with procedures in home energy rating systems (HERS) that offer a similar scale for measuring installation effectiveness. A Good rating for insulation installation indicates only occasional very small gaps are present and only minor compression. A Fair rating indicates moderate to frequent defects, but no more than 2% of the insulation is missing. A Poor rating is anything worse than Fair. The following guides may prove useful in determining a rating.

- http://www.homeenergy.org/show/article/id/35

- www.getenergysmart.com/Files/NewHomes/InsulationAssessment.doc

- http://www.nhec.com/filerepository/thermbypassguide.pdf

- http://ubees.utah.gov/documents/Summit/Utah%20Conference%20-%20Insulation%20Installation%20Presentation%20[Compatibility%20Mode].pdf

The grading scale associated with these more subjective requirements is used to adjust the points received for that code requirement. In an effort to focus on the most important code requirements, the checklist items have been clustered into multiple tiers. Each tier is given a different weight in determining the overall building compliance metric, with Tier 1 items receiving 3 points, Tier 2 items receiving 2 points, and Tier 3 items receiving 1 point.  For checklist items containing a grading scale, the overall points established by the tier are further modified based on the grading scale. A ‘Good’ rating will result in the checklist item being allocated all available points, a ‘Fair’ rating will result in an allocation of 2/3 of the available points, and a ‘Poor’ rating will result in an allocation of 1/3 of the available points. There are cases where a requirement with a grading scale does not comply altogether, in which case it should be marked as not compliant and it will receive 0 points.

| IECC Section # | Pre-Inspection/Plan Review |
| --- | --- |
| 103.1, 103.2 # [PR1]1 | Documentation. Determine if a complete set of plans/construction drawings, specifications, and energy code compliance documentation is available in the building department and documents building thermal envelope compliance. If there is no building department or the locality does not conduct plan review, this information should be obtained from the registered design professional or builder having responsibility for the project. If documentation indicating a trade-off or performance approach is not provided, a prescriptive approach must be assumed for verifying compliance. Construction documents should demonstrate energy code compliance with the building thermal envelope, including but not limited to the following information: - The location and R-values of insulation materials - U-factors and SHGC values for windows, doors, skylights, and other fenestration products Under the assumption that only state or local government with a responsible enforcement and/or permitting agency are included in compliance evaluations, plans and documentation are expected to be held by the responsible agency. If this is not the case, mark all Plan Review code requirements as non-compliant, unless there is another entity responsible for enforcement identified (e.g. utility, contractor licensing board, etc.), in which case they should be contacted to review this information. In rare cases where there is no jurisdictional oversight and builders self-certify, this requirement can be marked N/A. |
| 103.1, 103.2, 403.7 # [PR3]1 | Documentation. Determine if a complete set of plans/construction drawings, specifications, and energy code compliance documentation is available in the building department for determining lighting and mechanical system compliance. Construction documents should demonstrate energy code compliance with the building lighting and mechanical systems, including but not limited to: - Information related to duct and piping location, insulation type and R-value, and means of sealing - Type and capacity of mechanical system(s) - Efficacy of electrical lighting sources (lamps) - Mechanical ventilation system fan efficiency Mechanical systems serving multiple dwelling units must demonstrate compliance with the 2012 IECC Commercial Provisions instead of Section R403 of the 2012 IECC Residential Provisions. If there is no building department or the locality does not conduct plan review, this information should be obtained from the registered design professional or builder having responsibility for the project. Documents should sufficiently demonstrate energy code compliance for lighting and mechanical systems, including but not limited to information related to duct and piping location, duct and pipe insulation type and R-value, and means of sealing. |
| 302.1, 403.6 #### [PR2]2 | #### HVAC Equipment Correctly Sized. Verify that HVAC equipment is correctly sized according to ACCA Manual S based on building loads calculated according to Manual J or other methods approved by the code official. Verify the methodology used in the load calculations. List the resultant heating and/or cooling equipment capacities, as applicable, in the Plans Verified Value column. |

| IECC Section # | Foundation Inspection |
| --- | --- |
| 402.1.1 # [FO1]1 | # Slab Edge Insulation R-Value. Determine and record the R-value of slab insulation from the label on the insulation or from manufacturer shipping materials available onsite. Slabs are required to be insulated where the floor surface is less than 12 in. below grade. Slab insulation must be inspected prior to pouring the slab—when the insulation installation is completely visible. If the area has been designated as having heavy termite infestation, then insulation is not required and should be so noted. If slab edge insulation is required, but none is found, the following checklist items would all be marked non-compliant: • Slab Edge Insulation R-Value • Slab Edge Insulation Installation • Slab Edge Insulation Depth/Length |
| 303.2, 402.2.9 # [FO2]1 | # Slab Edge Insulation Installation. Insulation location can be vertical or horizontal inside the foundation wall, however, it must start at the top surface the slab and extend downward to completely cover the slab edge. It can also be located outside the foundation wall. Where insulation is located outside the wall and where it extends horizontally away from the building, it must be protected by pavement or at least 10 in. of soil. Verify that insulation is installed according to manufacturer’s instructions. |
| 402.1.1 [FO3]1 | Slab Edge Insulation Depth/Length. Measure and record the length of the slab insulation from the top of the insulation, which must be at the same level as the top of the floor slab, vertically and/or horizontally along the insulation path. The insulation application must be continuous in order to comply with the code. |
| 402.1.1 [FO4]1 | Conditioned Basement Wall Insulation R-Value. Determine and record the R-value of insulation applied to a wall associated with a conditioned basement or a basement that is unconditioned but does not have the floor above and other components separating the basement from the rest of the building insulated as part of the building thermal envelope. A basement wall is one that is at least 50% below grade. Where exterior insulation is applied, this inspection must be done immediately prior to backfilling when the insulation installation is completely visible. If interior insulation is applied, verification may need to be done during Insulation Inspection, in which case this checklist item may be left blank until that inspection stage. Basement wall insulation is not required in warm, humid locations as defined in Table R301.1 of the IECC. If basement insulation is required, but none is found, the following checklist items would all be marked non-compliant: • Basement Wall Insulation R-Value • Basement Wall Insulation Installation • Basement Wall Insulation Depth |
| 303.2 [FO5]1 | Basement Wall Insulation Installation. Verify that the insulation is installed according to manufacturer’s instructions. |
| 402.2.8 [FO6]1 | Basement Wall Insulation Depth. Measure and record the length of basement wall insulation from the top of the basement wall to the basement floor or until the insulation stops. For the prescriptive approach, the insulation length must be the lesser of 10 feet or to the top of the basement floor. For alternative approaches, verify the installation by reviewing the energy code compliance documentation. |
| 402.2.10 [FO7]1 | Unvented Crawl Space Wall Insulation R-Value. Determine and record the R-value of insulation applied to the interior or exterior of walls associated with crawl spaces that are not ventilated to the outside. Insulation installed on the exterior of the foundation wall must be inspected when the insulation is completely visible; immediately prior to backfilling. Insulation installed on the interior of the foundation wall will typically be inspected during the insulation inspection, in which case this checklist item may be left blank until that inspection stage. If crawl space wall insulation is required, but none is found, the following checklist items would all be marked non-compliant: • Crawl Space Wall Insulation R-Value • Crawl Space Wall Insulation Installation • Crawl Space Vapor Retarder - Crawl Space Wall Insulation Depth |
| 303.2 [FO8]1 | Unvented Crawl Space Wall Insulation Installation. Insulation must be installed according to manufacturer’s instructions. If the crawl space is ventilated, the floor above the crawl space must be insulated instead of insulating the crawl space walls. |
| 402.2.10 [FO9]1 | Unvented Crawl Space Vapor Retarder. Where a crawl space is unvented (e.g. not open to the building exterior) a Class I vapor retarder must be applied to the entire floor and run at least 6 in. up the walls of the crawl space and sealed to the walls. A Class I vapor retarder has a perm rating of less than 0.1 perm (such as polyethylene). If the product is not readily identified as to its perm rating then ask for supporting information or record the manufacturers information and validate it later. Any seams in the vapor retarder must have an overlap of at least 6 in. and be sealed or taped. |
| 402.2.10 [FO10]1 | Crawl Space Wall Insulation Depth. Verify that crawl space wall insulation extends downward from the floor to the finished grade, then vertically and/or horizontally for at least an additional 24 in. |
| 303.2.1 [FO11]2 | Insulation Protection. Determine that all slab, basement wall, or crawl space insulation exposed to the outside is protected from damage by an opaque covering that extends a minimum of 6 in. below grade. |
| 403.8 [FO12]2 | Snow Melt. If the building is provided with a snow or ice melting system supplied through the building energy service, verify that the system has controls to automatically shut the system off when the pavement is above 50 ºF and no precipitation is falling, and also manual controls or automatic controls that control shutoff when the outdoor temperature is above 40 ºF. Verification can be through direct inspection of the controls or, if not already installed, then verification that the system being installed is listed and labeled as having such controls. |

| IECC Section # | Framing / Rough-In Inspection |
| --- | --- |
| 402.1.1, 402.3.4 [FR1]1 | Door U-Factor. Determine and record the U-factor(s) for the door assemblies installed in the building thermal envelope. This information should be available from a label applied to the assembly, from packaging associated with the product or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. Under the prescriptive approach only, up to 24 ft2 of side-hinged door need not meet the specified U-factor in the code. Indicate in the comments column the total area of any non-complying products. |
| 402.1.1, 402.3.1, 402.3.3, 402.3.6, 402.5 [FR2]1 | Glazing U-Factor. Determine and record the U-factor(s) for the window, door, and glass block assemblies installed in the building thermal envelope that are not skylights, and excluding glazing installed in a sunroom that is thermally isolated from the rest of the building. Skylights are defined as installed glazing with a slope less than 60 degrees from horizontal. This information should be available from the National Fenestration Rating Council (NFRC) label applied to the assembly, from packaging associated with the product, or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. If default U-factors were used from Table 303.1.3 of the IECC, verify the frame type found in the field matches those on the approved plans. An area-weighted average can be used to satisfy the U-factor requirement. For the prescriptive approach only, up to 15 ft2 of the total glazed fenestration, including skylights, need not meet the specified U-factor in the code. |
| 402.1.1, 402.3.2, 402.3.3, 402.3.6, 402.5 [FR3]1 | Glazing SHGC Values. Determine and record the Solar Heat Gain Coefficient (SHGC) for the window, door, and glass block assemblies installed in the building envelope that are not skylights. Skylights are defined as installed glazing with a slope less than 60 degrees from horizontal. Glazing installed in a sunroom is subject to this requirement. This information should be available from the NFRC label applied to the assembly, from packaging associated with the product or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. If default SHGC values were used from Table 303.1.3 of the IECC, verify the glazing type (clear or tinted) found in the field match those on the approved plans. An area-weighted average can be used to satisfy the U-factor requirement. For the prescriptive approach only, up to 15 ft2 of glazed fenestration, including skylights, need not meet the specified SHGC requirement. Indicate in the comments column the total area of any non-complying products. SHGC requirements apply to Climate Zones 1-3 and Climate Zone 4 except Marine. |
| 303.1.3 [FR4]1 | Glazing Labeled for U-factor. Determine if fenestration U-factors are labeled and certified as meeting the referenced NFRC standards. If not, verify that compliance was based on the default U-factors from Table 303.1.3 of the IECC. Fenestration includes skylights, roof windows, vertical windows, opaque and glazed doors, and glass block. |
| 402.1.1, 402.3.3, 402.3.6, 402.5 [FR5]1 | Skylight U-Factor. Determine and record the U-factor for skylights and roof windows (e.g., glazing with a slope less than 60 degrees from horizontal) installed in the building envelope, but excluding skylights installed in a sunroom that is thermally isolated from the rest of the building. This information should be available from an NFRC label applied to the assembly, from packaging associated with the product, or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. If default U-factors were used from Table 303.1.3 of the IECC, verify the frame type found in the field match those on the approved plans. For the prescriptive approach only, up to 15 ft2 of the total glazed fenestration, including skylights, need not meet the specified U-factor in the code. |
| 402.1.1, 402.3.3, 402.3.6, 402.5 [FR6]1 | Skylight SHGC Values. Determine and record the SHGC for skylights and roof windows (e.g., glazing with a slope less than 60 degrees from horizontal) installed in the building thermal envelope, but excluding skylights installed in a sunroom that is thermally isolated from the rest of the building. This information should be available from the NFRC label applied to the assembly, from packaging associated with the product or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. If default SHGC values were used from Table R303.1.3 of the IECC, verify the glazing type (clear or tinted) found in the field match those on the approved plans. For the prescriptive approach only, up to 15 ft2 of glazed fenestration need not meet the specified SHGC requirement. Indicate in the comments column the total area of any non-complying products. |
| 303.1.3 [FR7]1 | Glazing Labeled for SHGC. Determine if glazed fenestration SHGC values are labeled and certified as meeting the referenced NFRC standards. If not, verify that compliance was based on the default SHGC values from Table R303.1.3 of the IECC. SHGC requirements apply to Climate Zones 1-3 and Climate Zone 4 except Marine. Glazed fenestration includes skylights, roof windows, vertical windows, glazed doors, and glass block. |
| 402.1.1 [FR10]1 | Mass Wall Exterior Insulation R-Value. A mass wall is an above-grade wall of concrete block, concrete, insulated concrete form (ICF), masonry cavity, brick (other than brick veneer), earth (adobe, compressed earth block, rammed earth) and solid timber/logs. Determine and record the R-value(s) of insulation applied to mass walls associated with the building thermal envelope other than basement walls. An above-grade wall is one that is less than 50% below grade. This inspection must be done prior to application of interior finish and after service systems are installed within the wall and/or before exterior finish that would hide the insulation and prohibit complete inspection of the installation. If insulated concrete forms are used, consult the manufacturer’s specified R-value for the product. If more than half of the insulation is on the interior side of the mass wall, the mass wall interior insulation requirement applies (IN3), and this requirement should be marked N/A. |
| 303.2 [FR11]1 | Mass Wall Insulation Installation. Determine that mass wall insulation is installed in accordance with the manufacturer’s installation instructions and all places in or on the mass wall that will accommodate insulation are insulated. Verify that the insulation installation instructions are available onsite or are readily available to the insulation contractors/installers. |
| 402.3.5 [FR8]1 | Sunroom Windows and Doors. Sunrooms are defined as one-story structures, attached to a dwelling unit, surrounding conditioned space, and with a gross glazing area greater than 40% of the sunroom’s exterior walls and roof. Verify that the sunroom glazing has a maximum U-factor of 0.40 in Climate Zone 2, 0.35 in Climate Zones 3 and 4 (except Marine), and 0.32 in Climate Zones 4 (Marine) through 8. Where the sunroom is thermally isolated from the rest of the dwelling unit, the maximum glazing U-factor in Climate Zones 4-8 is 0.45. New fenestration separating a thermally isolated sunroom from the dwelling unit must meet the thermal building envelope code requirements. This information should be available from the National Fenestration Rating Council (NFRC) label applied to the assembly, from packaging associated with the product, or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. If default U-factors were used from Table R303.1.3 of the IECC, verify the frame type found in the field matches those on the approved plans. |
| 402.3.5 [FR9]1 | Sunroom Skylight U-factor. Sunrooms are defined as one-story structures, attached to a dwelling unit, surrounding conditioned space, and with a gross glazing area greater than 40% of the sunroom’s exterior walls and roof. Verify that sunroom skylights have a maximum U-factor of 0.75 in Climate Zone 1, 0.65 in Climate Zone 2 and 0.55 in Climate Zones 3-8. Where the sunroom is thermally isolated from the rest of the dwelling unit, the maximum skylight U-factor is 0.70. This information should be available from the National Fenestration Rating Council (NFRC) label applied to the assembly, from packaging associated with the product, or by recording the manufacturer make and model number and frame type and looking up the information from the manufacturer’s web site or www.nfrc.org. If default U-factors were used from Table R303.1.3 of the IECC, verify the frame type found in the field matches those on the approved plans. |
| 402.4.1.1 [FR23]1 | Air Barrier. The air barrier, insulation, and other components of the building thermal envelope defined in Table R402.4.1.1 of the IECC must be installed according to manufacturer’s instructions and the criteria in Table R402.4.1.1 of the IECC. |
| 402.4.3 [FR20]1 | Fenestration Air Leakage. Inspect each window, skylight, and door to validate it satisfies the required air leakage rate (site-built windows, skylights, and doors are exempted). Compliance with air infiltration limits can often be evaluated based on a label on the product from an approved third party agency. The air infiltration rates in the code are the maximum permitted by standard AAMA/WDMA/CSA 101/I.S.2/A440-08. Any window that is listed and labeled as complying with this standard meets the infiltration requirement. If not listed and labeled to that standard NFRC 400 may have been used as the test standard and a label should be available that indicates the tested air leakage rate for the product. Note that NFRC 400 is simply a test method and does not indicate whether the product meets the code infiltration limits. If neither of the above-mentioned labels can be found, it may be possible to ask the builder or contractor for information validating compliance, and/or to get the make and model number of the product and consult the manufacturer’s listing. In cases where the evaluator knows that products were labeled, but those labels were removed prior to an onsite visit, an alternative building should be used to check this requirement. |
| 402.4.4 [FR16]2 | IC-Rated Recessed Lighting Fixtures. Identify all recessed lighting fixtures installed in the building thermal envelope. Verify that they are rated for installation in areas with insulation (e.g. IC rated), have a label on them from an approved third party indicating that they have an air leakage rate not exceeding 2 cfm at 75 Pa (1.57 psf) when tested under ASTM E283 and have a gasket or caulk applied between the fixture housing and the interior finish of the space where they are located. |
| 403.2.1 [FR12]1 | Duct Insulation. Determine and record the R-value(s) of insulation applied to, or used in, the manufacture of heating and/or cooling ducts that are not completely inside the building thermal envelope (e.g., are located outside the conditioned space). Supply ducts in attics must be insulated to at least R-8, and all other ducts in unconditioned spaces or not completely inside the building thermal envelope must be insulated to at least R-6. If all ducts are located completely within conditioned spaces, mark this requirement as compliant. If all systems are ductless, mark this requirement N/A. |
| 403.2.2 [FR13]1 | Duct Sealing. Verify that all ducts, air handlers, and filter boxes have joints and seams sealed. This should be verified before application of any duct insulation and before the ducts are made inaccessible for inspection by subsequent construction. Where flexible ducts containing insulation as an integral component of the duct are installed, the inspection for duct sealing must be conducted before the connections of the flex duct are taped. Approved duct sealing methods are provided in Section M1601.4.1 of the International Residential Code (IRC) and should be consulted for one- and two-family dwellings and townhouses. For multi-family dwellings, the provisions of the International Mechanical Code (IMC) should be consulted. |
| 403.2.3 [FR15]3 | Building Cavities as Supply Ducts. Verify that no building framing cavities are used as supply ducts or plenums (e.g., function to actually form the duct or plenum and move environmental air to/from conditioned spaces and HVAC equipment). |
| 403.3 [FR17]2 | HVAC Piping Insulation. Determine and record the R-value(s) of insulation applied to HVAC system piping, regardless of location, and the design temperature of the fluids being conveyed in the piping. For central air-conditioning systems and split-system heat pumps, HVAC piping refers to the refrigerant piping. This inspection must be done prior to application of additional pipe coatings or subsequent construction that would make the piping inaccessible for inspection. HVAC piping that could convey fluids above 105 ºF or below 55 ºF must be insulated to at least R-3. |
| 403.3.1 [FR24]2 | Protection of Piping Insulation. Verify that exposed piping insulation is protected from damage caused by sun, moisture, equipment maintenance, and wind. Protective measures should field from solar radiation that can degrade the insulation. Adhesive tape is not allowed. |
| 403.4.2 [FR18]2 | Hot-Water Piping Insulation. Determine and record the R-value(s) of insulation applied to service hot water piping, regardless of location. This inspection must be done prior to application of additional pipe coatings or subsequent construction that would make the piping inaccessible for inspection. All pipes should be insulated to at least R-3 or meet the run length requirements in Table R403.4.2 of the IECC. |
| 403.5 [FR19]2 | Outdoor Intake/Exhaust Openings. Identify all outdoor intake and exhaust openings and verify that they have either manual (self-closing) or automatic dampers that will close when the system associated with the air intake or exhaust is not functioning. This inspection should be done by cycling each affected system and observing damper operation. |

| IECC Section # | Insulation Inspection |
| --- | --- |
| 303.1 [IN13]2 | Insulation R-values. Verify that all insulation installed in the building thermal envelope has a label on the insulation indicating the R-value of the insulation or the insulation installer has provided a certificate verifying the type of insulation, the installed thickness and installed R-value. In addition, a certificate for blown in insulation must provide the installed density, coverage and number of bags of insulation. |
| 402.1.1, 402.2.6 [IN1]1 | Floor Insulation R-Value. Determine and record the R-value(s) of insulation applied to any wood-framed, steel-framed, or raised (e.g., not slab on grade) concrete floor associated with the building thermal envelope. If continuous insulation is installed, record the R-value of foam board insulation from the label on the insulation or from manufacturer shipping materials available onsite. This inspection must be done prior to completion of subsequent construction that would make the insulation inaccessible. |
| 303.2, 402.2.7 [IN2]1 | Floor Insulation Installation. Verify that floor insulation is installed in accordance with the manufacturer’s installation instructions, that all places in the floor that will accommodate insulation are insulated, and that the insulation is installed in direct contact with the underside of the subfloor decking. Verify the installation instructions are onsite or are readily available to the insulation contractors/installers. |
| 402.1.1, 402.2.5, 402.2.6 [IN3]1 | Wall Insulation R-Value. Determine and record the R-value(s) of insulation applied to wood-framed, steel-framed, and mass walls that are above grade and associated with the building thermal envelope. An above-grade wall is one that is more than 50% above grade. Mass walls are those of concrete block, concrete, ICFs, masonry cavity, brick (non-veneer), earth/adobe, and solid timber/logs. If at least half of the insulation on a mass wall is on the exterior side of the mass wall, the mass wall exterior insulation requirement applies (FR10), and this requirement should be marked N/A. If continuous insulation is installed, record the R-value of foam board insulation from the label on the insulation or from manufacturer shipping materials available onsite. This inspection must be done prior to application of interior finish and after service systems are installed within the wall and/or before exterior finish that would hide the insulation from inspection. |
| 303.2 [IN4]1 | Wall Insulation Installation. Determine that wall insulation is installed in accordance with the manufacturer’s installation instructions and all places in the wall that will accommodate insulation are insulated. Verify the insulation installation instructions are onsite or are readily available to the insulation contractors/installers. If the insulation is integral to a masonry wall (e.g., applied to concrete masonry unit open areas or integral to insulated concrete forms), verify that the insulation is uniformly applied throughout the wall. |
| 402.2.12 [IN8]1 | Sunroom Wall Insulation R-Value. Sunrooms are defined as one-story structures, attached to a dwelling unit, surrounding conditioned space, and with a gross glazing area greater than 40% of the sunroom’s exterior walls and roof. Verify that the sunroom walls meet the wall insulation requirements (IN3) unless the sunroom is thermally isolated from the rest of the dwelling, in which case the sunroom walls must be insulated to at least R-13 and the walls separating the sunroom from the conditioned spaces in the dwelling unit insulated in accordance with the wall insulation requirements (IN3). This inspection must be done prior to application of interior finish and after service systems are within the wall and/or before exterior finish that would hide the insulation from inspection. |
| 303.2 [IN9]1 | Sunroom Wall Insulation Installation. Determine that wall insulation applied to sunroom walls is installed in accordance with the manufacturer’s installation instructions and that all places in the wall that will accommodate insulation are insulated. Verify that instructions are onsite or are readily available to the insulation contractors/installers. If the insulation is integral to a masonry wall (e.g., applied to CMU open areas or integral to insulated concrete forms), verify that the insulation is uniformly applied throughout the wall. |
| 402.2.11 [IN10]1 | Sunroom Ceiling Insulation R-Value. Sunrooms are defined as one-story structures, attached to a dwelling unit, surrounding conditioned space, and with a gross glazing area greater than 40% of the sunroom’s exterior walls and roof. Verify that the sunroom ceiling meets the ceiling insulation requirements (FI1) unless the sunroom is thermally isolated from the rest of the dwelling, in which case the sunroom ceiling must be insulated to at least R-19 in Climate Zones 1-4 and R-24 in Climate Zones 5-8. This inspection must be done prior to application of interior finish and after service systems are within the ceiling. |
| 303.2 [IN11]1 | Sunroom Ceiling Insulation Installation. Determine that insulation applied to sunroom ceilings is installed in accordance with the manufacturer’s installation instructions and all places in the ceiling that will accommodate insulation are insulated. Verify that the instructions are onsite or are readily available to the insulation contractors/installers. |

| IECC Section # | Final Inspection |
| --- | --- |
| 402.1.1, 402.2.1, 402.2.2, 402.2.6 [FI1]1 | Ceiling Insulation R-Value. Determine and record the R-value(s) of insulation applied to any framed ceiling. For blown-in attic insulation, verify that the thickness of the insulation is written on markers at a rate of one every 300 ft2 of attic space and compare this with the insulation certificate. This inspection must be done prior to application of interior finish and after service systems installed within the ceiling and/or before exterior finish that would hide the insulation from inspection. If credit is taken for a raised heel, energy or oversized truss rafter system, verify such a system has been installed. |
| 303.1.1.1, 303.2 [FI2]1 | Ceiling Insulation Installation. Determine that insulation is installed in accordance with the manufacturer’s installation instructions. The thickness, in inches, of blown-in or sprayed insulation must be marked at every 300 ft2 throughout the attic. |
| 402.2.3 [FI22]2 | Eave Baffles. In vented attics, a baffle comprised of any solid material is required adjacent to soffit and eave vents. The baffles must maintain an opening at least the size of the vent, and extend over the top of the attic insulation. |
| 402.2.4 [FI3]1 | Attic Access Insulation. Determine and record the R-value(s) of insulation applied to any attic access hatches and doors. |
| 402.4.1.2 [FI17]1 | Air Sealing Blower Door Test. Determine compliance with the air sealing requirements via testing. Record when the test was conducted (after rough-in and installation of penetrations), the leakage rate from the test, and the specifications under which the test was administered. This information should be available on a test report by the entity conducting the test. Blower door test results are to be conducted at 50 Pascals, and cannot exceed 5 air changes per hour in Climate Zones 1 and 2, and 3 air changes per hour in Climate Zones 3-8. |
| 402.4.2 [FI8]2 | Wood Burning Fireplaces. Verify that all fireplaces have tight fitting flue dampers and outdoor air for combustion. |
| 403.2.2 [FI4]1 | Duct Tightness Testing (Rough-In or Post-Construction). Verify that the ducts and air handler, if not completely located inside the conditioned spaces, were tested for tightness. Record the leakage rate from the test, and the specifications under which the test was administered (across the system or without the air handler installed). This information should be available on a test report by the entity conducting the test. Duct tightness test results cannot exceed 4 cfm/100 ft2 across the system or 3 cfm/100 ft2 without the air handler when tested at 25 Pascals. Verification performed via a rough-in test may need to occur during Framing Inspection. |
| 403.2.2.1 [FI24]1 | Sealed Air Handler. Air handlers must have an air leakage rate per ASHRAE Standard 193 of no more than 2% of the design air flow, and be as designated by the manufacturer. |
| 403.6 [FI5]1 | Heating and Cooling Equipment. Identify each piece of heating, cooling and heating/cooling equipment by manufacturer name and model number. Where multiple components are provided (e.g., split system heat pump or gas furnace/electric air conditioner) the model number of each component should be recorded. This information should be verified against the information identified from the plans and specifications. If there are no plans and specifications or those available do not match the submitted specifications then mark this item as non-complying. |
| 403.1.1 [FI9]2 | Programmable Thermostat. Verify that each dwelling unit has a programmable thermostat for each forced air furnace that can control the heating and cooling system to allow heating temperatures down to 55 °F and cooling temperatures at least 85 °F. |
| 403.1.2 [FI10]2 | Heat Pump Thermostat. Verify that heat pumps have a thermostat that will prevent backup heating from operating when the heating load can be satisfied by the heat pump. |
| 403.4.1 [FI11]2 | Service Water Piping System Controls. Identify all pumps associated with the circulating service hot water system and verify that they have an automatic or readily accessible manual controls that can turn the pump off when the system in not in use. This inspection should be done by loading (e.g., turning on) each affected system and observing pump operation. |
| 403.5.1 [FI25]2 | Ventilation Fan Efficacy. Bathroom and utility room fans with an air flow rate from 10 to less than 90 cfm must have a minimum efficacy of 1.4 cfm/watt. All other mechanical and ventilation system fans must have a minimum efficacy of 2.8 cfm/watt. |
| 403.9.1 [FI12]3 | Swimming Pool Heaters. Verify that all pool heaters have an accessible on-off switch to allow turning off the heater without adjusting the thermostat setting. |
| 403.9.2 [FI19]3 | Swimming Pool Heater Timer Switches. Verify there are time switches that can automatically turn on and off pool heating equipment. |
| 403.9.3 [FI20]3 | Heated Pools and Inground Permanently Installed Spa Covers. Verify that a vapor retardant cover is provided for heated pools and permanently installed, inground spas. |
| 404.1 [FI6]1 | Lighting. Determine how many permanently installed lamps there are in the dwelling unit and how many of those have high efficacy lamps (>=60 lumens per watt when over 40 watts, 50 lumens for over 15 to 40 watts, and 40 lumens for 15 watts or less) or how many permanent lighting fixtures there are in the dwelling unit and how many of those have high efficacy lamps. Under the prescriptive or trade-off approach, at least 75% of the individual lamps must be high-efficacy lamps or 75% of the fixtures must have high-efficacy lamps to be deemed compliant. If the building complies via the performance approach, mark this requirement N/A. |
| 401.3 [FI7]2 | Compliance Certificate. Verify that a certificate identifying the energy-related features of the building is located on or in the electrical panel, or other approved location. |
| 303.3 [FI18]3 | Manufacturer Manuals. Verify that manufacturer manuals for mechanical and water heating systems have been provided to the owner. This is a requirement that third-party evaluators may be unable to verify, in which case it can be marked Not Observable (N/O). |

| KEY | 1 | High Impact (Tier 1) | 2 | Medium Impact (Tier 2) | 3 | Low Impact (Tier 3) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

The 2012 IECC prefaces all section numbers in the residential provisions with an ‘R’, which has been left off in the checklists but is assumed in code references.

A document titled Measuring State Energy Code Compliance was developed by the U.S. Department of Energy’s Building Energy Codes Program (BECP), which included procedures and recommendations for computing a state compliance rate. The document is available at http://www.energycodes.gov/sites/default/files/documents/MeasuringStateCompliance.pdf.