03USC12/10/08

Section 565. — DRILLED SHAFTS

| Include the following when work is required in this Section. | (WFL Specification 12/10/08) |
| --- | --- |

Construction Requirements

565.06 Delete the second and third paragraphs and substitute the following:

Install access tubes for each drilled shaft according to ASTM D 6760. Extend access tubes a minimum of 2 feet above shaft tops and a maximum of 3 inches above shaft bottom.

565.07 Concrete for Drilled Shafts. Delete the fifth paragraph.

56508 Integrity Testing. Amend as follows:

Delete the text of the paragraph and substitute the following:

(a) Testing. Perform integrity testing on production drilled shafts according to ASTM D 6760. Test drilled shafts between 3 and 45 days after concrete placement. Provide drilled shaft bottom and tip elevations, access tube lengths and surveyed positions to the crosshole sonic logging subcontractor before testing. Perform tests between adjacent perimeter access tubes and diagonally between tubes within the drilled shafts. Test all possible cross-sections between access tubes.

If any of the access tubes are not plumb, retain water, tube to concrete debonding has occurred, or a tube obstruction exists, provide a core hole to replace the rejected tube or propose an alternative integrity-test method that is approved by the CO.

Fill access tubes with neat cement grout according to Subsection 725.22(f) after integrity testing, inspection, the data has been analyized and accepted by the CO. Use a tremie beginning at the bottom of the tube during grouting.

Delete the second sentence in the first paragraph of (b).