03USC06/04/07

Section 706. — CONCRETE AND PLASTIC PIPE

| Include the following when precast reinforced concrete box sections are required. | FLH Transmittal No. 3 06/04/07 |
| --- | --- |

706.07 Precast Reinforced Concrete Box Sections. Delete the text of this Subsection and substitute the following:

Conform to ASTM C 1433M. Meet the design requirements for HS20 loading.

| Include the following when underdrain pipe less than or equal to 10 inches is required. | (WFL Specification 01/01/04) |
| --- | --- |

706.08 Plastic pipe. Delete the text of paragraph (d) and substitute the following:

(d) Corrugated polyethylene drainage tubing. Furnish 3- to 10-inch diameter tubing conforming to AASHTO M 252, Type C or CP.